Prilog 2: Tehničke specifikacije

Naziv predmeta nabave: 	Sustav za čišćenje sa prednje 2 četke

Evidencijski broj nabave: 18-11.18

Zahtjevi definirani Tehničkim specifikacijama predstavljaju minimalne tehničke karakteristike odnosno standarde koje ponuđeni predmet nabave mora zadovoljavati.

Ponuditelj popunjava samo ćelije obilježene sivom bojom.

Stupac "NAPOMENA" ponuditelj može popuniti ukoliko smatra potrebnim.

Kako bi se ponuda smatrala valjanom, ponuđeni predmet nabave mora zadovoljiti sve što je traženo u obrascu Tehničkih specifikacija.

Za predmet nabave, za sve (pod)stavke/opise/upućivanja na mjesto u kojima se eventualno traži ili navodi marka, patent, tip, norma ili određeno podrijetlo, ponuditelj može ponuditi „jednakovrijedno“ traženom ili navedenom, te će Naručitelj prihvatiti i druge jednakovrijedne mjere osiguranja kvalitete, ali u tom slučaju mora uz ponudu priložiti dokaze o jednakovrijednosti (katalog, potvrde proizvođača ili sl.). „Jednakovrijedno“ je sve ponuđeno što nije unutar propisanog opisa, ali zadovoljava minimalne tehničke karakteristike predložene (pod)stavke. Gdje je primjenjivo, karakteristike moraju odgovarati traženom uz odstupanje do +/- 1%, ukoliko raspon nije drugačije definiran od strane Naručitelja. Izuzev kod oblikovnih karakteristika, Naručitelj će prihvatiti i veći postotak odstupanja ukoliko isto predstavlja više standarde, odnosno tehničke karakteristike koje su bolje od propisanih minimalnih.

| R.BR. | NAZIV STAVKE | TRAŽENE TEHNIČKE SPECIFIKACIJE / FUNKCIONALNOSTI |  | PONUĐENO | NAPOMENA (ukoliko je potrebno) |
| --- | --- | --- | --- | --- | --- |
|  |  | NAZIV/OPIS PODSTAVKE |  |  |  |
| 1. | Sustav za čišćenje sa prednje 2 četke: 1 komplet | 1.1. Četka čelična sa plastičnom prevlakom: 14 kom |  |  |  |
|  |  | Model: | Weber circular brush Silent ili jednakovrijedno |  |  |
|  |  | Unutarnji promjer tijela četke: | 100 mm |  |  |
|  |  | Vanjski promjer tijela četke: | 500 mm |  |  |
|  |  | Materijal tijela četke: | drvo |  |  |
|  |  | Tip tijela četke: | jednodijelno tijelo |  |  |
|  |  | Vanjski promjer četke: | 850 mm |  |  |
|  |  | Materijal čekinja: | plosnata žica φ2,8mm sa plastičnom prevlakom |  |  |
|  |  | Broj redova čekinja: | 4 |  |  |
|  |  | 1.2. Četka čelična: 14 kom |  |  |  |
|  |  | Model: | Weber Multipurpose brush for machines up to 5 m3 ili jednakovrijedno |  |  |
|  |  | Unutarnji promjer tijela četke: | 100 mm |  |  |
|  |  | Vanjski promjer tijela četke: | 500 mm |  |  |
|  |  | Materijal tijela četke: | drvo |  |  |
|  |  | Tip tijela četke: | jednodijelno tijelo |  |  |
|  |  | Vanjski promjer četke: | 850 mm |  |  |
|  |  | Materijal čekinja: | plosnata žica φ2,8mm |  |  |
|  |  | Broj redova čekinja: | 4 |  |  |
|  |  | 1.3. Četka čelična + polimerna: 14 kom |  |  |  |
|  |  | Model: | Weber allrounder for machines up to 5 m3 ili jednakovrijedno |  |  |
|  |  | Unutarnji promjer tijela četke: | 100 mm |  |  |
|  |  | Vanjski promjer tijela četke: | 500 mm |  |  |
|  |  | Materijal tijela četke: | drvo |  |  |
|  |  | Tip tijela četke: | jednodijelno tijelo |  |  |
|  |  | Vanjski promjer četke: | 850 mm |  |  |
|  |  | Materijal čekinja 1: | plosnata žica φ2,8mm |  |  |
|  |  | Materijal čekinja 2: | polimerna žica promjera φ2,7mm, glatka, plava |  |  |
|  |  | Broj redova čekinja: | 4 |  |  |
|  |  | 1.4. Četka polimerna: 14 kom |  |  |  |
|  |  | Model: | Weber Plastic brush ili jednakovrijedno |  |  |
|  |  | Unutarnji promjer tijela četke: | 100 mm |  |  |
|  |  | Vanjski promjer tijela četke: | 500 mm |  |  |
|  |  | Materijal tijela četke: | drvo |  |  |
|  |  | Tip tijela četke: | jednodijelno tijelo |  |  |
|  |  | Vanjski promjer četke: | 850 mm |  |  |
|  |  | Materijal čekinja: | polimerna žica promjera φ2,7mm, glatka, plava |  |  |
|  |  | Broj redova čekinja: | 4 |  |  |

Annex 2: Technical specifications

Procurement title: 	Sweeping system with 2 front brushes

Procurement record number: 18-11.18

The requirements defined by the Technical Specifications represent the minimum characteristics and standards that the offered object of the procurement must meet.

Tenderer shall complete only cells marked with gray color.

The column "NOTES" tenderer can fill up if deemed necessary.

For offer to be consider as compliant, offered object of the procurement must meet all the requirements presented in the Technical Specifications.

For the subject of the procurement, for all (sub)items/descriptions/referring to the place which may be affixed to a trademark, patent, type, norm or specific origin, the Tenderer may offer "equivalent" to the requested or specified and Contracting Authority will accept other equivalent quality assurance measures, but in that case the Tenderer must enclose proof of equivalence (catalog, manufacturer or the like). "Equivalent" is all off offered that is not within the prescribed description but meets the minimum technical characteristics of the required (sub)items. Where applicable, the characteristics must correspond to the defined ones, with the deviation up to +/- 1%, unless the range is otherwise defined by the Contracting Authority. Except for the aesthetic characteristics, the Contracting Authority will accept a higher percentage of deviation if it represents a higher standard or technical characteristics that are better from the prescribed minimum.

| ITEM NO. | ITEM | REQUIRED TECHNICAL SPECIFICATIONS / FUNCTIONALITIES |  | OFFERED | NOTES (if deemed necessary) |
| --- | --- | --- | --- | --- | --- |
|  |  | SUBITEM NAME/DESCRIPTION |  |  |  |
| 1. | Sweeping system with 2 front brushes: 1 set | 1.1. Steel brush with plastic coating: 14 pcs |  |  |  |
|  |  | Model: | Weber circular brush Silent or equivalent |  |  |
|  |  | Brush body inner diameter: | 100 mm |  |  |
|  |  | Brush body outer diameter: | 500 mm |  |  |
|  |  | Brush body material: | wood |  |  |
|  |  | Brush body type: | one piece body |  |  |
|  |  | Brush outer diameter: | 850 mm |  |  |
|  |  | Bristles material: | flat wire φ2,8mm with plastic coating |  |  |
|  |  | Number of bristle rows: | 4 |  |  |
|  |  | 1.2. Steel brush: 14 pcs |  |  |  |
|  |  | Model: | Weber Multipurpose brush for machines up to 5 m3 or equivalent |  |  |
|  |  | Brush body inner diameter: | 100 mm |  |  |
|  |  | Brush body outer diameter: | 500 mm |  |  |
|  |  | Brush body material: | wood |  |  |
|  |  | Brush body type: | one piece body |  |  |
|  |  | Brush outer diameter: | 850 mm |  |  |
|  |  | Bristles material: | flat wire φ2,8mm |  |  |
|  |  | Number of bristle rows: | 4 |  |  |
|  |  | 1.3. Steel + polymer brush: 14 pcs |  |  |  |
|  |  | Model: | Weber allrounder for machines up to 5 m3 or equivalent |  |  |
|  |  | Brush body inner diameter: | 100 mm |  |  |
|  |  | Brush body outer diameter: | 500 mm |  |  |
|  |  | Brush body material: | wood |  |  |
|  |  | Brush body type: | one piece body |  |  |
|  |  | Brush outer diameter: | 850 mm |  |  |
|  |  | Bristles material 1: | flat wire φ2,8mm |  |  |
|  |  | Bristles material 2: | polymer wire diameter φ2,7mm, smooth surface, blue |  |  |
|  |  | Number of bristle rows: | 4 |  |  |
|  |  | 1.4. Polymer brush: 14 pcs |  |  |  |
|  |  | Model: | Weber Plastic brush or equivalent |  |  |
|  |  | Brush body inner diameter: | 100 mm |  |  |
|  |  | Brush body outer diameter: | 500 mm |  |  |
|  |  | Brush body material: | wood |  |  |
|  |  | Brush body type: | one piece body |  |  |
|  |  | Brush outer diameter: | 850 mm |  |  |
|  |  | Bristles material: | polymer wire diameter φ2,7mm, smooth surface, blue |  |  |
|  |  | Number of bristle rows: | 4 |  |  |